UCI DTN/HPC Filesystem and Network Performance

FIONA

CPU

less /proc/cpuinfo

- Intel(R) Xeon(R) CPU E5-1650 v3 @ 3.50GHz (12 cores, 15M cache)

Memory

RAM:

sudo dmidecode --type 17

sudo lshw -short -C memory

- 128GB System Memory (8 x 16GB DIMM Synchronous)

  - Size: 16384 MB

  - Speed: 2133 MHz

Storage and Filesystems:

sudo lshw -class disk

dmesg | less

sudo lshw | grep SAS (See disks listed under)

sudo lshw | grep SATA (See disks listed under)

- 16 x 240GB SSDS (Serial Attached SCSI controller, LSI Logic / Symbios Logic, SAS-3)

  - ZFS partition

  - SAS-3 has 12Gbps speed

sudo zpool status

There are two raidz1 virtual devices and one pool:

    - pool: bigdata

      - raidz1-0

      - raidz1-1

sudo df –h

    - bigdata         2.9T  700G  2.2T  25% /bigdata

sudo zpool iostat

Shows more accurate information on the pool allocations.

capacity     operations    bandwidth

pool        alloc   free   read  write   read  write

----------  -----  -----  -----  -----  -----  -----

bigdata      831G  2.66T      0     17  18.8K  11.4M

- 2 x 240GB SSDS (SATA controller, C610/X99 series chipset 6-Port SATA Controller)

  - XFS

  - Linux raid

cat /proc/mdstat

    - md126 : active raid1

    - md127 : active raid1

df –h

    - /dev/md127      223G   86G  137G  39% /

    - /dev/md126     1015M  224M  792M  23% /boot

Since the partition is a raid1, the two 240GB drives are mirrored.

sudo dmidecode -t 2

Manufacturer: Supermicro

Product Name: X10SRL-F

Googling the manufacturer and product name:

http://www.supermicro.com/products/motherboard/xeon/c600/x10srl-f.cfm

- 10x SATA3 (6Gbps) via C612

sudo lshw -C disk

product: INTEL SSDSC2BW24

Googling it up we see these benchmark tests:

http://ssd.userbenchmark.com/SpeedTest/3104/INTEL-SSDSC2BW240A4

Avg. Sequential Write Speed 225MB/s

So it seems that even though the SATA3 controller maxes out at 6Gbps, the SSD Hard Drives’s average sequential write speed is 225MB/s.

Is the maximum achievable write speed set by the individual disks or the SATA3 controller (if aggregate)?  How does the number of users affect these speeds?

Tests

DD Fun

time sh -c "dd if=/dev/zero of=/home/jtatar/10GB.test bs=1M count=10240 && sync"

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 4.65732 s, 2.3 GB/s

real	0m28.916s

user	0m0.006s

sys	0m6.034s

10737418240/28.916s/10^9 = 370MB/s

time sh -c "dd if=/dev/zero of=/home/jtatar/10GB.test bs=1M count=10240 conv=fdatasync"

gives the same result as && sync above

- Note: sync – forces the process to write the entire file to disk before completing. Note, that dd will return before completing but the time command will not, therefore the time output will include the sync to disk.

time sh -c "dd if=/dev/zero of=/bigdata/transfers/10GB.test bs=1M count=10240 && sync"

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 2.24651 s, 4.8 GB/s

real	0m8.365s

user	0m0.007s

sys	0m1.827s

1.3GB/s

time sh -c "dd if=/dev/zero of=/bigdata/transfers/10GB.test bs=1M count=10240 conv=fdatasync"

gives the same result as && sync above

Let’s sync to disk every 1M – this should be the slowest mode (lower threshold), as the write cache is basically unused:

time sh -c "dd if=/dev/zero of=/home/jtatar/10GB.test bs=1M count=10240 oflag=dsync"

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 152.497 s, 70.4 MB/s

real	2m33.895s

user	0m0.013s

sys	0m10.307s

70MB/s

time sh -c "dd if=/dev/zero of=/bigdata/transfers/10GB.test bs=1M count=10240 oflag=dsync"

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 87.6996 s, 122 MB/s

real	1m27.728s

user	0m0.011s

sys	0m4.805s

122MB/s

FIONA DD Fun Summary

Writing to /home/* (XFS mirror) is slower than /bigdata (ZFS) because of the filesystem

SATA controller upper bound: 6Gbps

Avg. SSD upper bound per channel: 225MB/s

DD && sync /bigdata: limits = (122MB/s, 1.3GB/s)

DD && sync /home: limits = (70MB/s, 350MB/s)

For fastest transfers, /bigdata should be used.

HPC Interactive Node

lshw -short -C memory

256GiB System Memory

8GiB DIMM DDR3 Synchronous 1600 MHz (0.6 ns)

lshw -class disk

size: 2794GiB (3TB)

lshw | grep SATA

SB7x0/SB8x0/SB9x0 SATA Controller [IDE mode]

dmesg | grep SATA

SATA link up 3.0 Gbps (SStatus 123 SControl 300)

df -H

Filesystem            Size  Used Avail Use% Mounted on

/dev/sda2             2.2T   12G  2.1T   1% /

tmpfs                 136G     0  136G   0% /dev/shm

nas-7-7.local:/data    16T  9.0T  7.1T  56% /data

beegfs_dfs1           685T  515T  170T  76% /dfs1

beegfs_dfs2           443T  409T   34T  93% /dfs2

beegfs_fast-scratch    14T  1.6T   12T  12% /fast-scratch

beegfs_sbak           597T  352T  245T  59% /sbak

compute-4-43.ib:/adl  111T   48T   64T  44% /share/adl

nas-7-2.ib:/pub        61T   53T  7.4T  88% /share/pub

nas-1-1.ib:/zfs//samdata

119T   91T   29T  77% /share/samdata

compute-2-13.ib:/jje   12T  9.5T  2.6T  79% /share/jje

So to test the local filesystem we should write to /scratch

/data – nas-7-7 (mounted over Ethernet)

/pub – nas-7-2 (over IB)

/ssd-scratch – nas-7-1

/fast-scratch – nas-7-1

/flash-scratch – nas-7-1 (not mounted on compute-1-13)

So nas-7-1 seems to be the fast moose in the forest.

lshw -short -C memory

512GiB System Memory

16GiB DIMM DDR3 Synchronous 1600 MHz

lshw | grep SAS

product: SAS2008 PCI-Express Fusion-MPT SAS-2 [Falcon]

product: MegaRAID SAS 2208 [Thunderbolt]

product: SAS2X36

product: SAS2X36

These are 6Gbps controllers.

lshw | grep SATA

description: SATA controller

product: SB7x0/SB8x0/SB9x0 SATA Controller [IDE mode]

DFS1 and DFS2

jtatar@compute-1-13/dfs1/jtatar$ time sh -c "dd if=/dev/zero of=/dfs1/jtatar/10GB.test bs=1M count=10240 && sync"

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 21.2095 s, 506 MB/s

real	0m23.144s

user	0m0.014s

sys	0m11.822s

jtatar@compute-1-13/dfs2/jtatar$ time sh -c "dd if=/dev/zero of=/dfs2/jtatar/10GB.test bs=1M count=10240 && sync"

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 12.9465 s, 829 MB/s

real	0m14.911s

user	0m0.011s

sys	0m6.416s

jtatar@compute-1-13/dfs1/jtatar$ time sh -c "dd if=/dev/zero of=/dfs1/jtatar/10GB.test bs=1M count=10240 conv=fdatasync && sync"

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 22.0205 s, 488 MB/s

real	0m24.603s

user	0m0.014s

sys	0m13.174s

jtatar@compute-1-13/dfs2/jtatar$ time sh -c "dd if=/dev/zero of=/dfs2/jtatar/10GB.test bs=1M count=10240 conv=fdatasync && sync"

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 23.2052 s, 463 MB/s

real	0m25.391s

user	0m0.014s

sys	0m7.178s

Where are the dfs1/dfs2 file servers:

less /etc/hosts

# BeeGFS Filesystems - Infiniband

10.2.255.160    nas-7-1.ib

10.2.255.153    dfm-1-1.ib

10.2.255.139    dfm-2-1.ib

10.2.254.164    dfs-3-1.ib

10.2.254.163    dfs-3-2.ib

Transferring the arianna SIM dataset:

globus-url-copy -vb -cd -r -sync -p 8 -s "/C=US/O=Globus Consortium/OU=Globus Connect Service/CN=bf20a3d8-880f-11e7-a95a-22000a92523b" -cd -r /pub/arianna/SIM/ gsiftp://jtatar@fiona.oit.uci.edu/bigdata/data/arianna/

While transfer is taking place, let’s take a look at what dfm-2-1 is doing:

ssh dfm-2-1

iftop –i ib0

compute-1-13 is 10.2.255.157

dfm-2-1.ib                   => 10.2.255.157                  445Kb   579Kb   646Kb

<=                               	     427Kb   555Kb   620Kb

Transferred 370GB from HPC to FIONA.  The peak rate was 1.36Gbps.  Transfer started 10:28am PST finished at 1:12pm PST.

Clocked the transfer speed at 11:30am and got ~100MB/s avg

Clocked the transfer speed at the end: 370000MB/(164*60)s = 40MB/s avg

After the first hour, the transfer rate dropped significantly.  Could this be because the file sizes decreased?  Yes, in part.

Let’s test setup end to end as a researcher would have to if their follow current instructions.  For these tests, let’s use 380GB of the ARIANNA experiment SIM data.

Since /dfs1 and /dfs2 are now the largest capacity file systems, I am guessing users won’t have to clean data they put on them:

Filesystem            	Size  Used Avail Use% Mounted on

/dev/sda2             	2.0T   11G  1.9T   1% /

tmpfs                 	127G  100K  127G   1% /dev/shm

nas-7-7.local:/data    	15T  8.2T  6.4T  57% /data

beegfs_dfs1           	623T  470T  153T  76% /dfs1

beegfs_dfs2           	403T  377T   26T  94% /dfs2

beegfs_fast-scratch    	13T  532G   12T   5% /fast-scratch

beegfs_sbak           	543T  348T  196T  65% /sbak

nas-7-2.ib:/pub        	55T   50T  5.5T  91% /share/pub

Otherwise, they would have to move their data from /pub to /dfs#.  Let’s see how long that takes:

jtatar@compute-1-13/dfs2/jtatar$ time cp -r /pub/arianna/SIM .

real	129m25.413s

user	0m3.156s

sys	9m30.080s

Wow.  It took 2h to move 380GB of data from /pub to /dfs2

When we take the above result and consider the result of the data transfer of the same dataset from HPC to FIONA it becomes clear that the /pub is the bottle neck.  What else have we learned?  /fast-scratch is not that fast.

Let’s confirm these suspicions and see how long it takes to transfer the data set from HPC /dfs2 to FIONA’s /bigdata.  I expect about 400MB/s.

──────────────────────────────────────────────────────────────────────────────────────

TX:             cum:   11.8MB   peak:	2.09Mb        rates:   1.78Mb  1.90Mb  1.77Mb

RX:                    14.3GB           2.60Gb                 1.86Gb  2.26Gb  2.15Gb

TOTAL:                 14.3GB           2.60Gb                 1.86Gb  2.26Gb  2.16Gb

──────────────────────────────────────────────────────────────────────────────────────

TX:             cum:   18.7GB   peak:   6.43Gb        rates:   1.43Mb  1.78Mb  3.24Gb

RX:                    55.3GB           2.56Gb                 2.56Gb  2.19Gb  2.17Gb

TOTAL:                 74.0GB           7.64Gb                 2.56Gb  2.20Gb  5.41Gb

250GB in 20 minutes or at about 210MB/s

For large files peak speed was about 350MB/s

300GB were transferred in 25 min

The last 80GB are small files around 300KBs.

──────────────────────────────────────────────────────────────────────────────────────

TX:             cum:   28.2GB   peak:   1.60Mb        rates:   1.60Mb  1.16Mb  1.23Mb

RX:                     148GB            171Mb                  171Mb   123Mb   130Mb

TOTAL:                  177GB            172Mb                  172Mb   124Mb   131Mb

──────────────────────────────────────────────────────────────────────────────────────

TX:             cum:   28.2GB   peak:   1.56Mb        rates:   1.17Mb  1.21Mb  1.22Mb

RX:                     149GB            166Mb                  124Mb   128Mb   128Mb

TOTAL:                  177GB            168Mb                  125Mb   129Mb   130Mb

Clocking at 8GB in 8min for around 17MB/s.

Final results: 370GB in 102min = 60MB/s

real	102m32.829s

user	2m44.064s

sys	13m23.580s

The 370GB ARIANNA test dataset we have been using consists of a variety of file sizes that range from 1.1GB on down to several KB.  The fast majority of the files ~300KB:

du -ah /bigdata/data/boo/DTN_Tests | grep -v "/$" | sort -rh

1.1G    ./CoREAS_May2015/229

1.1G    ./CoREAS_May2015/200/DAT000001

1.1G    ./CoREAS_May2015/200

1.1G    ./CoREAS_July2016/2195/DAT000001

1.1G    ./CoREAS_July2016/2195

1.1G    ./CoREAS_Feb2016/544

1021M   ./CoREAS_June2015/310

1000M   ./CoREAS_Feb2016/544/DAT000001

992M    ./CoREAS_Feb2016/542

983M    ./CoREAS_May2015/229/DAT000001

976M    ./CoREAS_June2015/310/DAT000001

967M    ./CoREAS_May2015/182

960M    ./CoREAS_June2015/348

947M    ./CoREAS_Feb2016/542/DAT000001

946M    ./CoREAS_June2015/382

932M    ./CoREAS_May2015/209

922M    ./CoREAS_May2015/182/DAT000001

918M    ./CoREAS_June2015/483

915M    ./CoREAS_June2015/348/DAT000001

902M    ./CoREAS_June2015/382/DAT000001

887M    ./CoREAS_May2015/209/DAT000001

883M    ./CoREAS_May2015/218

883M    ./CoREAS_June2015/339

873M    ./CoREAS_June2015/483/DAT000001

866M    ./CoREAS_Feb2016/549

859M    ./CoREAS_June2015/394

850M    ./CoREAS_Feb2016/545

838M    ./CoREAS_May2015/218/DAT000001

838M    ./CoREAS_June2015/339/DAT000001

837M    ./CoREAS_May2015/136

836M    ./CoREAS_June2015/323

827M    ./CoREAS_May2015/125

821M    ./CoREAS_Feb2016/549/DAT000001

820M    ./CoREAS_May2015/193

818M    ./CoREAS_May2015/156

814M    ./CoREAS_June2015/394/DAT000001

813M    ./CoREAS_June2015/340

810M    ./CoREAS_July2016/1919

808M    ./CoREAS_July2016/2199

808M    ./CoREAS_July2016/2196

805M    ./CoREAS_Feb2016/545/DAT000001

803M    ./CoREAS_June2015/381

792M    ./CoREAS_May2015/136/DAT000001

…etc..

7.0K	./CoREAS_August2017_HCR/2502/steering/SIM000001.reas

7.0K	./CoREAS_August2017_HCR/2502/steering/RUN000001.inp

7.0K	./CoREAS_August2017_HCR/2501/steering/SIM000001.reas

7.0K	./CoREAS_August2017_HCR/2501/steering/RUN000001.inp

7.0K	./CoREAS_August2017_HCR/2500/steering/SIM000001.reas

7.0K	./CoREAS_August2017_HCR/2500/steering/RUN000001.inp

1.5K	./CoREAS_July2016/2226

1.5K	./CoREAS_July2016/2223

1.5K	./CoREAS_July2016/2222

1.5K	./CoREAS_July2016/2221

1.5K	./CoREAS_July2016/2194

1.0K	./CoREAS_July2016/2229

1.0K	./CoREAS_July2016/2228

1.0K	./CoREAS_July2016/2227

1.0K	./CoREAS_July2016/2226/steering

1.0K	./CoREAS_July2016/2223/steering

1.0K	./CoREAS_July2016/2222/steering

1.0K	./CoREAS_July2016/2221/steering

1.0K	./CoREAS_July2016/2220

1.0K	./CoREAS_July2016/2219

1.0K	./CoREAS_July2016/2218

1.0K	./CoREAS_July2016/2217

1.0K	./CoREAS_July2016/2205

1.0K	./CoREAS_July2016/2198

1.0K	./CoREAS_July2016/2194/steering

1.0K	./CoREAS_July2016/2193

512	./SimFromJames/outdata/day_99

512	./SimFromJames/outdata/day_97

512	./SimFromJames/outdata/day_96

512	./SimFromJames/outdata/day_93

512	./SimFromJames/outdata/day_92

512	./SimFromJames/outdata/day_81

512	./SimFromJames/outdata/day_8

Let’s transfer this data to NERSC.  Executing on the FIONA:

myproxy-logon -b -T -l jtatar -s nerscca.nersc.gov

time globus-url-copy -vb -cd -r -p 8 file:///bigdata/data/boo/ gsiftp://dtn01.nersc.gov/global/u2/j/jtatar/DTN_Tests/

Reached the 50GB quota on the NERSC DTN.  Let’s find another filesystem to write to…

Let’s try writing to the cori scratch filesystem:

time globus-url-copy -vb -cd -r -p 8 file:///bigdata/data/boo/ gsiftp://corigrid.nersc.gov/global/cscratch1/sd/jtatar/DTN_Tests/

real	249m38.422s

user	4m17.832s

sys	5m20.170s

jtatar@cori17:/global/cscratch1/sd/jtatar> du -hs

381G	.

[jtatar@fiona ~]$ du -hs /bigdata/data/boo/

370G	/bigdata/data/boo/

Weird, does ZFS compress the data?  Is that why it is 370G on FIONA but 381GB on Cori?

Well, that’s pitiful: 25MB/s upload from FIONA to Cori at NERSC.

Now pulling data by initiating the transfer from the FIONA by downloading the data we stored at NERSC.  On the FIONA is is 370GB/282min = 22MB/s

real    282m52.008s

user    5m25.621s

sys     5m50.605s

We should try this with Globus…

NERSC to FIONA:  110MB/s

FIONA to NERSC: 175MB/s

Let’s double check the file speed of large files by transferring 4 files equaling ~380GB total.  Creating 3 different 100GB files and a 80GB file:

time sh -c "dd if=/dev/zero of=/bigdata/data/boo/BigFiles_test/fl1_100GB.test bs=1M count=102400 && sync"

File 1:

102400+0 records in

102400+0 records out

107374182400 bytes (107 GB) copied, 70.1362 s, 1.5 GB/s

real	1m25.613s

user	0m0.057s

sys	0m18.980s

File 2:

102400+0 records in

102400+0 records out

107374182400 bytes (107 GB) copied, 70.1698 s, 1.5 GB/s

real	1m22.648s

user	0m0.053s

sys	0m20.383s

File 3:

102400+0 records in

102400+0 records out

107374182400 bytes (107 GB) copied, 70.051 s, 1.5 GB/s

real	1m25.611s

user	0m0.060s

sys	0m19.215s

time sh -c "dd if=/dev/zero of=/bigdata/data/boo/BigFiles_test/80GB.test bs=1M count=80240 && sync"

File 4:

80240+0 records in

80240+0 records out

84137738240 bytes (84 GB) copied, 53.2722 s, 1.6 GB/s

real	1m7.832s

user	0m0.050s

sys	0m15.339s

FIONA to NERSC = 330MB/s

NERSC to FIONA = 350MB/s

Silly to do transfers to FIONA just to be able to initiate a GridFTP session from outside UCI.  Let’s mount /dfs1 and /dfs2 on FIONA by installing the BeeGFS client.

sudo wget -O /etc/yum.repos.d/beegfs-rhel7.repo https://www.beegfs.io/release/latest-stable/dists/beegfs-rhel7.repo

sudo yum install beegfs-client beegfs-helperd beegfs-utils

sudo vim /etc/beegfs/beegfs-client-autobuild.conf

Let’s try it with default kernel IB modules:

buildArgs=-j8 BEEGFS_OPENTK_IBVERBS=1

“Note: In the buildArgs example above, OFED include files are installed to /usr/src/openib. If you are using the default kernel IB modules (i.e. not the separate OFED modules), then the option OFED_INCLUDE_PATH does not need to be set at all.”

sudo /etc/init.d/beegfs-client rebuild

Quick test of the read speed:

FIONA /bigdata:

[jtatar@fiona BigFiles_test]$ dd if=80GB.test of=/dev/null bs=1M count=80240

80240+0 records in

80240+0 records out

84137738240 bytes (84 GB) copied, 46.1888 s, 1.8 GB/s

[jtatar@fiona BigFiles_test]$ dd if=80GB.test of=/dev/null bs=1M count=10240

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 7.94453 s, 1.4 GB/s

HPC DFS:

jtatar@compute-1-13/dfs2/jtatar$ dd if=10GB.test of=/dev/null bs=1M count=10240

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 20.5664 s, 522 MB/s

jtatar@compute-1-13/dfs1/jtatar$ dd if=10GB.test of=/dev/null bs=1M count=10240

10240+0 records in

10240+0 records out

10737418240 bytes (11 GB) copied, 15.2723 s, 703 MB/s